SUB.  :  CONSTRUCTION OF TABLET DUST COLLECTOR ROOM AT THE FACTORY OF GLUCONATE HEALTH LIMITED AT 1, HEALTH INSTITUTE ROAD, KOLKATA-700065
Sealed quotations are invited from Reliable, Resourceful, Bonafide and Experienced Civil contractors for the following  job as per the schedule given.

 TENDER  NO.     :    GHL/CW-04/16-17   DATE  :  30.07.2016.

  Last Date and Time of  Submission  of  Quotations  :  On 16.08.2016 up to 2.00 pm                                                                     

           Date and Time of opening of quotations : On 16.08.2016 at 3.00 pm

           Quotations submitted later than 2.00 pm of 16.08.2016 will not be accepted.

           Please submit the quotations at the Purchase Dept of our Head Office at 2, Durga Charan     

           Doctor  Lane, Kolkata – 700 014  within the schedule date and time.
	
	
	
	
	
	
	

	1
	
	SCHEDULE OF WORK.

Earth work in excavation of foundation trenches or drains, in all sorts of soil (including mixed soil but excluding laterite or sandstone) including removing, spreading or stacking the spoils within a lead of 75m. as directed. The item includes necessary trimming the sides of trenches, levelling, dressing and ramming the bottom, bailing out water as required complete. Stripping off worn out plaster and raking out joints of walls, ceilings etc. upto any height and in any floor including removing rubbish within a lead of 75m. Depth of excavation not exceeding 1,500mm.
	%Cum
	0.71
	
	

	2
	
	Single Brick Flat Soling of picked jhama bricks including ramming and dressing bed to proper level and filling joints with local sand.
	Sqm
	0.97
	
	

	3
	
	Ordinary Cement concrete (mix 1:2:4) with graded stone chips (20 mm nominal size) excluding shuttering and reinforcement,if any, in ground floor as per relevant IS codes. a) Pakur Variety 
	Cum
	0.097
	
	

	4
	
	Hire and labour charges for shuttering with centering and necessary staging upto 4m using approved stout props and thick hard wood planks of approved thickness with equired bracing for concrete slabs, beams and columns, lintels curved or straight including fitting, fixing and striking out after completion of works (upto roof of ground floor) (a) 25 mm to 30 mm thick wooden shuttering as per decision & direction of Engineer-In-Charge.
	Sqm
	0.708
	
	

	5
	
	Brick work with 1st class bricks in cement mortar (1:4), ground floor
	Cum
	0.86
	
	

	6
	
	125 mm. thick brick work with 1st class bricks in cement mortar (1:3) in ground floor. 
	Sqm
	5.00
	
	

	7
	
	Filling in foundation or plinth by silver sand in layers not exceeding 150 mm as directed and consolidating the same by thorough saturation with water, ramming complete including the cost of supply of sand. (payment to be made on measurement of finished quantity)
	%Cum
	0.62
	
	

	8
	
	Ordinary Cement concrete (mix 1:1.5:3) with
graded stone chips (20 mm nominal size)
excluding shuttering and reinforcement if
any, in ground floor as per relevant IS codes. Pakur Variety
	Cum
	0.15
	
	

	9
	
	Reinforcement for reinforced concrete work in all sorts of structures including distribution bars, stirrups, binders etc initial straightening and removal of loose rust (if necessary), cutting to requisite length, hooking and bending to correct shape, placing in proper position and binding with 16 gauge black annealed wire at every intersection, complete as per drawing and direction. For works in foundation, basement and upto roof of ground floor/upto 4m. Tor steel/Mild Steel. 
	MT
	0.016
	
	

	10
	
	Plaster (to wall, floor, ceiling etc.) with sand and cement mortar including rounding off or chamfering corners as directed and raking out joints including throating, nosing and drip course, scaffolding/staging where necessary (Ground floor).[Excluding cost of chipping over concrete surface], With 1:4 cement mortar, 15mm thick plaster. 
	Sqm
	    15.22 
	
	

	11
	
	Wood work in posts, post plates, rafters, battens, truss members, purlins etc. fitted and fixed complete (excluding the cost of bolts, paints, but including the cost of nails, screws etc.) (The quantum should be corrected upto three decimals). Sal : Siliguri.
	Cum
	0.014
	
	

	12
	
	Galvanised corrugated iron sheet work (excluding the supporting frame work) fitted and fixed with 10 mm. dia J or L hook-bolts, limpet and bitumen washers and putty complete with 150 mm. end  lap and one corrugation minimum side lap. (Payment to be made on area of finished work)(GCI sheet to be supplied by contractor), In Roof, With 0.80 mm thick sheet. 
	Sqm
	2.82
	
	

	13
	
	Artificial stone in floor, dado, staircase etc with cement concrete (1:2:4) with stone chips, laid in panels as directed with topping made with ordinary or white cement (as necessary) and marble dust in proportion (1:2) including smooth finishing and rounding off corners including raking out joints or roughening of concrete surface and application of cement slurry before flooring works using cement @ 1.75 kg/sq.m all complete including all materials and labour. In ground floor. 3 mm. thick topping (High polishing grinding on this item is not permitted with ordinary cement). Using grey cement, 35mm. thick 
	Sqm
	2.42
	
	

	14
	
	Neat cement punning about 1.5mm thick in
wall,dado,window sill,floor etc. NOTE:Cement 0.152 cu.m per100 sq.m.
	Sqm
	1.92
	
	

	15
	
	Priming one coat on timber or plastered
surface with synthetic oil bound primer of
approved quality including smoothening
surfaces by sand papering etc.
	Sqm
	25.45
	
	

	16
	
	Painting with best quality synthetic enamel paint of approved make and brand including smoothening surface by sand papering etc. including using of approved putty etc. on the surface, if necessary: On timber or plastered surface: With super gloss (hi-gloss), Two coats (with any shade except white)
	Sqm
	25.45
	
	

	17
	
	M.S.or W.I. Ornamental grill of approved design joints continuously welded with M.S, W.I. Flats and bars of windows, railing etc. fitted and fixed with necessary screws and lugs in ground floor. The weight of grill per sq.m. will be determined by taking the physical weight of fabricated grill and dividing the same by covered area of the same.Grill weighing above 16Kg./sq.mtr and above.
	Qtl
	1.210
	
	

	18
	
	Priming one coat on steel or other metal surface with synthetic oil bound primer of approved quality including smoothening surfaces by sand papering etc. 
	Sqm
	3.45
	
	

	19
	
	Painting with best quality synthetic enamel paint of approved make and brand including smoothening surface by sand papering etc, on steel or other metal surface With super gloss (hi-gloss), Two coats (with any shade except white) 
	Sqm
	3.45
	
	


TERMS & CONDITIONS :

1.  INSPECTION OF SITE AND STUDY OF LOCAL CONDITIONS

Intending tenderers are required to inspect the site of work with particular reference to location and infrastructural facilities. They are to make a careful study with regard to availability of materials and their sources, labour (skilled and unskilled) and all relevant factors as might affect their rates and prices. A tenderer who submits a tender will be deemed to have inspected the site and make proper study of all relevant factors. No extra payment for mobilisation advance and price escalation will be allowed at any point of time during execution of the work. 
2.  The  successful  tenderer  shall  have  to  submit i)  Performance Bank Guarantee @ 5 % of the order value in the form of Bank Guarantee or Demand Draft, to be kept with the company till successful completion and hand over of the project, duly certified by our authorized person/s, and ii) Security Deposit @ 5% of the order value which will be kept with the company after deducting from the running bills, and the total amount will be refunded after completion of the guarantee period.  
3.  SUBMISSION OF TENDER

 The Tenderer is required to carefully study all the TERMS & CONDITIONS and prepare his Tender to comply with all provisions thereof. Submission of a Tender shall be taken as an evidence and confirmation that the Tenderer has acknowledged all the provisions of the Tender Documents and has fully acquainted himself with site conditions and all factors which may influence the preparation of his Tender. Negligence of the Tenderer to observe instructions in preparing his Tender shall be at his own risk and shall not be a ground for securing relief from any error found or discrepancies contained in his Tender or a cause for withdrawal of his Tender after it has been opened.

All expenses incurred for site inspection and all activities in the preparation and submission of the Tender shall be borne by the Tenderer.


All rates and prices shall be written in figures as well as in words. In case of any discrepancy between the two, words shall prevail over the figures.

4.  SCHEDULE OF PRICES

The Tenderer shall submit the rate against individual items clearly. The rates against different items should be reasonable and commensurate with one another. Any rate which, in the opinion of the authority accepting tenders, is absolutely high or absolutely low in comparison to the quoted rates of other items, may lead to rejection of the Tender. 

5.  VALIDITY OF TENDER

A Tender submitted shall remain valid for a period of One Hundred and Twenty (120) calendar days from the date set for opening of tenders. Any extension of this validity period will be subject to concurrence of the Tenderer.

6.  WITHDRAWAL OF TENDER

A Tender once submitted shall not be withdrawn within the validity period.However a tenderer may withdraw his tender by writing an application to TENDER INVITING AUTHORITY before the Scheduled opening time of the tenders.

7.  OPENING AND PRELIMINARY EXAMINATION OF TENDERS

Immediately after the time stupultated  for receipt of tenders, all tenders received shall be opened by TENDER INVITING AUTHORITY or his authorised representative in the presence of such tenderers or their authorised representatives as may be present or be represented at the time of opening of tenders.

Contents of the individual tenders shall first be examined in a general way as to their conformity and agreement with the tendering requirements. Any tender not conforming to any of the requirements may be disqualified forthwith at the discretion of the authority opening tenders. Tenders not so disqualified shall be subject to detailed scrutiny as there under.

8.  ACCEPTANCE OF TENDER

Tender will be accepted (for and on behalf of the TENDER INVITING AUTHORITY). He does not bind himself to accept the lowest tender and reserves to himself the right to reject any or all of the tenders received without assigning any reason therefor.

The work to be completed within 1 (One) calendar  month  from the date of work order issued, under the guidance of the Works Manager of the company and the guarantee period from the date of official completion and hand over of the project will be for a span of 12 (Twelve) calendar  months.

9.  AGREEMENT                                                                                                                                                                          The successful Tenderer shall be required to enter into a contract through  agreement if so requires by GHL. 

10.  COMPLETION & PENALTY                                                                                                                                             The job must be completed within the stipulated time as per the terms of the tender and work 
order & the agreement to be signed. The contractor must submit a detail working chart to the company along with the quotation showing the complete details. The relevant drawings are also to be submitted.                                                 

Penalty: For delay in completion beyond stipulated time, an amount equal to 0.5% of the total value of the order for each day’s  delay will be deducted from the contractor’s bill at the 
discretion of Gluconate Health Limited. Management of GHL also reserves the right to cancel the order for non completion of work within the stipulated time and get the work done by other parties and debit the difference if any, between tendered and fresh rates to the account of the Tenderer. The security deposit paid will also be forfeited by GHL due to non-fulfillment or non-performance of the contract in part or full.

11.  Materials:  The tenderer must ensure about the quality of the materials supplied and to be used only after certified by the Works Manager of the company.

Tenderers are advised to consult with manufacturer/  experts at his own cost, if so felt, to reach more Correct figure for Tendering purpose.

12.  Payment Terms: No advance payment will be made. Payments will be made against submission of running bills after deducting the security deposit @ 5% . Final payment will be made within 60 days on completion of job, properly certified by the competent authority and hand over of the site to the concerned department. Security deposit will be refunded after completion of Guarantee period which will be not less than one year from the date of the hand over of the project.

Purchase Manager

Gluconate Health Limited

STATUTORY / MISC. LIABILITIES OF THE CONTRACTOR.
1. Contractor agrees to indemnify the Management (i.e. Principal Employer) that it will be solely and entirely responsible to comply with and abide by all the statutory provision as and when applicable under various labour and Industrial Law enacted by the Government like contract Labour (Regulation & Abolition) Act 1970; payment of wages act 1936, Factories Act 1948; Provident Fund Act 1952; Employees State Insurance Act. Minimum Wages Act, Workmen Compensation Act 1923 & rules with subsequent amendments in force at that time. 

2. Contractor’s undertakes the responsibility that it shall pay wages to his contractor labour’s every month in due time.

3. Contractor shall maintain registers & recorder as required under various labour & industrial laws & the rules made there under as & when applicable. Agency shall furnish these registers & records to the Management when ever asked for. 

4. Contractor shall “Indemnify” GHL against all claims which may be made under the above said acts on accounts of any compensation payable to contractor’s labour whether  in the employment of contractor or not . Similarly contractor shall ensure that working hours, fixation of wages, working condition are to be followed as stipulated in the license. 

5. If under any circumstances what so ever GHL is held responsible in any manner what so ever for payment of compensation to contractor’s laborer for default or omission of commission of contractor, then GHL is entitled to be reimbursed by the contractor along with all expenses, costs claims etc. 

6. Contract work will be supervised by a technically sound person of the contractor.

7. Contractor shall be fully responsible for the safety of his workmen / employees. In any case, it shall be the sole responsibility of the contractor to per liability / compensation in the event of contractor while working at GHL Premises. 

8. Contractor shall strictly follow the prevailing rules and regulations of GHL. 

Signature of Contractor with seal & date. 
