                                                                                                                                              Phone     : 2265-0001 (3 Lines)

                                                                                                                                              Fax          : (033) 2265-8537

                                                                                                                                              Website   :  www.gluconatehealth.com
                                                                                                                                              E-mail      : ghlmd@rediffmail.com  

	[image: image1.jpg]



	     Gluconate Health Limited

               (A  GOVERNMENT OF WEST BENGAL UNDERTAKING)

                    Registered & Head Office: 2, DURGA CHARAN DOCTOR LANE,  KOLKATA – 700 014 


[image: image1.jpg]



 S.O.W

Name of Work: Emergent overhauling/Maintenance of one no. 500 KVA, 50 H3 Truvol make transformer and 6.6 KV, 400A Bieco Lawrie make HT Breaker including S&F of 250A PDB panel (For Tablets Dept. Service Floor), Maintenance of TPN main switches at GHL Factory.

Code No. :
	Sl. No
	Description of item
	Qnty.
	Rate
	Amount

	1
	Replacement of the damaged HV & MV side bushing including bought out neutral bushing of the 500 KVA transformers.
(Ref: W|O No: 17/EE(E)EBSD/EM/KMDA/Q-02, Dt. 25.07.2018)
	
1 Job
	
	

	2
	Replacement of both side terminal box, gasket breather, silica gel along with conductor etc. of the 500 KVA transformer.
(Ref: W|O No: 17/EE(E)EBSD/EM/KMDA/Q-02, Dt. 25.07.2018)
	
1 Job
	
	

	3
	Repairing rectification installation as commissioning of Buchoiz relay of the 500 KVA transformers.
(Ref: W|O No: 17/EE(E)EBSD/EM/KMDA/Q-02, dt. 25.07.2018)
	
1 Job
	
	

	4
	Installation of 500 KVA Transformer by modification of the pedestal as tit-bit repairing of the 500 KVA transformer.

(Ref: W|O No: 17/EE(E)EBSD/EM/KMDA/Q-02, dt. 25.07.2018)
	
1 Job
	
	

	5
	Supply & delivery of transformer oil 500 KYA grade as per IE rule.
(Ref: W|O No: 17/EE(E)EBSD/EM/KMDA/Q-02, dt. 25.07.2018)
	
60 Lts.
	
	

	6
	Dehydration and toping up of 500 KVA Transformer oil with dislodging by centrifugation method.
(Ref: W|O No: 17/EE(E)EBSD/EM/KMDA/Q-02, dt. 25.07.2018)
	
1 Job
	
	

	7
	HV testing of the 500 KVA transformer including HT breaker.
(Ref: W|O No: 17/EE(E)EBSD/EM/KMDA/Q-02, dt. 25.07.2018)
	

1 Job
	
	

	8
	Repairing & maintenance of Bieco Lawrie make HT, OCB including cleaning the circuit side & busbar side of spout and centoels as per instruction of EIC.
	
1 Job
	
	

	9
	Eassan Reyrolle relay testing and calibration type 2TJM10 to HT 6.6 KV OCB Bieco Lawrie make breaker.
	
1 Job
	
	

	10
	Supply, fitting & fixing of resin cast HT current transformer of ratio 100/50/5/5A, class 1/5P10. Type “S”, VA-15
	
3 Nos.
	
	

	11
	a) Supply and fixing double door vertical TPN MCB distribution board for MCCB incomer with IP-42/43 protection on angle iron frame on wall mending good the damage to origiual finish incl. inter connection with suitable size of copper wire and neutral link & provision for earthing attachment.
upto 160A-8-way. Mk: Legrand.
(Ref: PWD Sch. 2017, Pg. No: D-11, 
Sl. No. 15)


b) S&F 240/415V DP MCB module of on din rail of existing DB’s endosure and necessary connection
Rating – 201A DP MCB
Mk: Legrand.
(Ref: PWD Sch. 2017, Pg. No: D-7, 
Sl. No. 9)

c) S&F 240/415V MCB of breaking capacity 10 KA & C-characteristics on din rail of existing DB’s and necessary connection.
6 - 32A MCB – SPN. Mk: Legrand.
(Ref: Ref: PWD Sch. 2017, Pg. No: D-6, 
Sl. No. 7)


d) S&F of copper wire of size.
i)   2.5 sq. mm.’FR’ standard copper wire
ii)  1.5 sq. mm.’FR’ standard copper wire
iii) 4.0 sq. mm.’FR’ standard copper wire


e) S&F of MCCB thermal magnetic release range 160A – 200A, 415V 50 H3, Breaking capacity 36/50 KA. 4 – Pole,
Make: Legrand 
(Ref: Analysis Enclosed)


f) Supply & Delivery of spreader terminal for 4 – Pole MCCB
(Ref: Analysis Enclosed)
	
1 No.








8 Nos.

20 Nos.







50 mm.
50 mm.
50 mm.

1 No.

2 Set
	
	

	12
	S & D and fixing 2mm thick M.S. enclosure with angle frame of 1000 mm. X 750mm. X 250mm. 16 SWG sheet. (single door, IP-65 Model) .  
Ref. EE (E/M) SWTP/Q-23(E) OF 2016-17), adjusting tax.
	1 No.
	
	

	13
	S & F selector switch for the L.T. panel. Make Kaycee/ equivv. Ref.  EE(E/M) SWTP/Q-23(E)OF 2016-17, adjusting tax.
	2 Nos.
	
	

	14
	S & D LED indicator lamp colour RYB Cat no. LED2 X RLED X GLED. Make : GE/ equivalent.

 Ref.  EE(E/M) SWTP/Q-23(E)OF 2016-17, adjusting tax.
	3 Nos.
	
	

	15
	S & F of CT having ratio 100 : 54 or specified by E.I.C. or ring type CT for panel. Make:- L & T / Equivalent.

Ref.  EE(E/M) SWTP/Q-23(E)OF 2016-17, adjusting tax
	3 Nos.
	
	

	16
	S & D Ammeter ( 0-100) or specified by E.I.C. Make:- AE/ Equivalent.

Ref.  EE(E/M) SWTP/Q-23(E)OF 2016-17, adjusting tax
	2 Nos.
	
	

	17
	S & D of Voltmeter (0-500) V. 
Make: AE/Equivalent. 

Ref: EE (E/M) SWTP/Q-23(E) OF 2016-17, adjusting tax.
	
2 Nos.
	
	

	18
	S&D of MCCB of 200-250A thermal realease range, 4 – Pole, AC, 50HZ, Breaking Capacity 36/50Ka.
Make: Legrand


	
1 No.
	
	

	19
	S&D 240/415V MCB of breaking capacity 10Ka and C Characteristics on Din rail of existing DB’s and necessary connection 
i) 63A, 4 – Pole ….
ii) 100A, 4 – Pole….  
	


4 Nos.
2 Nos.
	
	

	20
	Supply and fixing of heavy duty backlite terminal block for 3-ph. Power line terminal for panel.
	7 Nos.
	
	

	21
	Modification, testing, re-wiring and inter connection of central panel.
	1 No.
	
	

	22
	S & F compression type gland complete with brass gland, brass ring & rubber ring for dust & moisture proof entry of XLPE/PVC armoured cable as below.

a) 3.5 core 120sq.mm. 
b) 3.5 core 50 sq.mm.
	2 Nos.
1 No.
	
	

	23
	Finishing of the XLPE/PVC insulated AR Cable ends by soldering with sockets and insulated etc. including supplying sockets, soldering materials etc. and making connection suttle, BDB and BBC etc.
a) 3.5 core 120sq.mm.
b) 3.5 core 50 sq.mm.
	2 Sets
1 Set
	
	



